# High-sensitivity nanoscale chemical imaging with hard x-ray nano-XANES

A. Pattammattel et al. (None)
DOI: 10.1126/sciadv.abb3615
Category: electrochemistry | Element: Fe | Edge: K-edge
4 condition(s)

Please review each condition and mark Correct / Incorrect.

# Condition 1: Physically mixed stainless steel and hematite nanoparticles

| Category | electrochemistry |
| --- | --- |
| Phases & Fractions | metallic_iron: 0.96, hematite: 0.04 |
| Reasoning | The sample is an ex-situ physical mixture prepared specifically as a reference standard to benchmark the sensitivity of the XANES technique to trace phases. Because it is a physical mixture and not subjected to electrochemical cycling, the phases remain exactly as prepared: predominantly metallic iron (from the stainless steel matrix) with a trace amount (4%) of hematite (Fe2O3). Therefore, only the metallic iron phase exceeds the 5% significance threshold. |
| Correct? | Yes / No — Comments: |

Stage 1 Prompt (what the LLM sees):

## Sample Information

- **Material**: Physically mixed stainless steel and hematite nanoparticles

- **Application/Context**: Benchmark reference standard for XANES analysis

## Electrode / Cell Details

- **Electrode type**: not reported

- **Active material**: Stainless steel and hematite

- **Electrolyte**: not reported

- **Cell type**: not reported

## Electrochemical Conditions

- **Charge/discharge state**: not reported

- **C-rate**: not reported

- **Voltage**: not reported

- **State of charge**: not reported

- **Cycle number**: not reported

- **Capacity**: not reported

## Sample-Specific Conditions

- Key variable: Sample type is a reference standard

- Measurement mode: ex-situ

- Physically mixed stainless steel and hematite nanoparticles used as a benchmark

## Measurement

We plan to measure the Fe K-edge XANES spectrum of this electrode.

## Questions

Based on ALL the electrochemical conditions above:

1. What phases (oxidation states / compounds of Fe) could exist in this electrode at this state?

2. How many of these phases are significant (>5% fraction)?

3. For each significant phase, estimate its fraction (fractions should sum to 1.0).

4. What reference spectra should be prepared for linear combination fitting?

5. What scientific reasoning connects the specific electrochemical conditions to your predicted phases and fractions?

# Condition 2: Carbon-coated lithium iron phosphate (LiFePO4) particles

| Category | electrochemistry |
| --- | --- |
| Phases & Fractions | LiFePO4: 0.95, Fe3P: 0.05 |
| Reasoning | The sample is pristine LiFePO4, meaning it has not been electrochemically cycled (SOC = pristine, cycle = 0). Therefore, the primary phase is the fully lithiated Fe(II) state, LiFePO4. The high-temperature carbon coating process typically reduces a small amount of the material to form trace secondary phases like iron phosphides (e.g., Fe3P), which account for the ~5% minority phase. |
| Correct? | Yes / No — Comments: |

Stage 1 Prompt (what the LLM sees):

## Sample Information

- **Material**: Carbon-coated lithium iron phosphate (LiFePO4) particles

- **Application/Context**: Lithium-ion batteries

## Electrode / Cell Details

- **Electrode type**: cathode

- **Active material**: LiFePO4

- **Electrolyte**: not reported

- **Cell type**: not reported

## Electrochemical Conditions

- **Charge/discharge state**: OCV

- **C-rate**: not reported

- **Voltage**: not reported

- **State of charge**: pristine

- **Cycle number**: 0

- **Capacity**: not reported

## Sample-Specific Conditions

- State of charge: pristine

- Measurement mode: ex-situ

- The sample is a pristine carbon-coated particle before any electrochemical cycling.

## Measurement

We plan to measure the Fe K-edge XANES spectrum of this electrode.

## Questions

Based on ALL the electrochemical conditions above:

1. What phases (oxidation states / compounds of Fe) could exist in this electrode at this state?

2. How many of these phases are significant (>5% fraction)?

3. For each significant phase, estimate its fraction (fractions should sum to 1.0).

4. What reference spectra should be prepared for linear combination fitting?

5. What scientific reasoning connects the specific electrochemical conditions to your predicted phases and fractions?

# Condition 3: LixFePO4

| Category | electrochemistry |
| --- | --- |
| Phases & Fractions | Fe(II)/Fe(III) states: 0.96, Fe3P: 0.04 |
| Reasoning | At a partially (de)lithiated state within the 3.0 - 4.0 V window, the active material undergoes a two-phase reaction, consisting predominantly of a mixture of LiFePO4 (Fe2+) and FePO4 (Fe3+), which together account for 96% of the iron species. The remaining 4% consists of trace Fe3P, a secondary phase originally formed during high-temperature carbon coating, which transforms into a percolating nanonetwork during cycling but remains a minor fraction overall. |
| Correct? | Yes / No — Comments: |

Stage 1 Prompt (what the LLM sees):

## Sample Information

- **Material**: LixFePO4

- **Application/Context**: Lithium-ion battery

## Electrode / Cell Details

- **Electrode type**: cathode

- **Active material**: LixFePO4

- **Electrolyte**: LiPF6 in dimethyl carbonate

- **Cell type**: CR-2032 coin cell

## Electrochemical Conditions

- **Charge/discharge state**: partially charged/discharged

- **C-rate**: 0.1 C

- **Voltage**: 3.0 - 4.0 V

- **State of charge**: partially (de)lithiated

- **Cycle number**: not reported

- **Capacity**: not reported

## Sample-Specific Conditions

- State of charge: partially (de)lithiated

- ex-situ

- LFP particle after partial (de)lithiation showing phosphide nanonetworks

## Measurement

We plan to measure the Fe K-edge XANES spectrum of this electrode.

## Questions

Based on ALL the electrochemical conditions above:

1. What phases (oxidation states / compounds of Fe) could exist in this electrode at this state?

2. How many of these phases are significant (>5% fraction)?

3. For each significant phase, estimate its fraction (fractions should sum to 1.0).

4. What reference spectra should be prepared for linear combination fitting?

5. What scientific reasoning connects the specific electrochemical conditions to your predicted phases and fractions?

# Condition 4: LixFePO4 with trace Fe-phosphide secondary phases

| Category | electrochemistry |
| --- | --- |
| Phases & Fractions | Fe3P: 0.18, LixFePO4 (Fe2+/Fe3+): 0.82 |
| Reasoning | During electrochemical cycling (partial de/lithiation) between 3.0 and 4.0 V, the main active material LixFePO4 exists as a mixture of Fe(II) and Fe(III) states. The trace Fe-phosphide (Fe3P) secondary phases, originally formed during high-temperature carbon coating, transform from isolated nanoparticles into percolating nanonetworks during cycling. In specific particles, this transformation leads to a locally high concentration of the phosphide phase (18%), with the remainder being the mixed-valence active material. |
| Correct? | Yes / No — Comments: |

Stage 1 Prompt (what the LLM sees):

## Sample Information

- **Material**: LixFePO4 with trace Fe-phosphide secondary phases

- **Application/Context**: Lithium-ion battery

## Electrode / Cell Details

- **Electrode type**: cathode

- **Active material**: LixFePO4

- **Electrolyte**: LiPF6 in dimethyl carbonate

- **Cell type**: CR-2032 coin cell

## Electrochemical Conditions

- **Charge/discharge state**: partially charged/discharged

- **C-rate**: 0.1 C

- **Voltage**: 3.0 - 4.0 V

- **State of charge**: partially (de)lithiated

- **Cycle number**: not reported

- **Capacity**: not reported

## Sample-Specific Conditions

- State of charge: partially (de)lithiated

- ex-situ

- Particle 2, selected to investigate trace Fe-phosphide secondary phases formed during carbon coating and their transformation during cycling

## Measurement

We plan to measure the Fe K-edge XANES spectrum of this electrode.

## Questions

Based on ALL the electrochemical conditions above:

1. What phases (oxidation states / compounds of Fe) could exist in this electrode at this state?

2. How many of these phases are significant (>5% fraction)?

3. For each significant phase, estimate its fraction (fractions should sum to 1.0).

4. What reference spectra should be prepared for linear combination fitting?

5. What scientific reasoning connects the specific electrochemical conditions to your predicted phases and fractions?

Overall comments on this paper: ________________________________________